SIMONE Forecast System

Gas Consumption Forecast
User Guide

April 2025

Version 1.0

# Table of Contents

Table of Contents will be generated here. In Microsoft Word, place cursor below and press Ctrl+F9, type "TOC \o \"1-3\" \h \z \u", press F9, then right-click and select "Update Field".

# 1. Overview

The SIMONE Forecast System is a machine-learning-based solution for predicting gas consumption across pipeline networks managed by the SIMONE simulation software. It combines historical consumption data extracted from SIMONE reconstruction scenarios with real-time weather data to generate accurate hourly gas demand forecasts.

The system is designed for gas transmission and distribution companies that use SIMONE for network simulation. By analyzing patterns in historical reconstruction data and correlating them with weather conditions (temperature, wind speed, humidity), the system produces forecasts that can be written back into SIMONE as forecast scenarios (FORECAST.RDF).

## 1.1 Architecture

The system consists of two main workflows:

train.py -- Extracts historical data from SIMONE reconstruction scenarios, downloads weather data, and trains a machine learning model. This is typically run once or periodically (e.g., weekly) to update the model with new data.

forecast.py -- Uses the trained model together with current weather forecasts to generate consumption predictions. It writes the forecast back into SIMONE as a new scenario. This is typically run daily, either manually or via batch automation.

Supporting modules:

config.py -- Centralized configuration with INI file support and environment variable overrides.

lang.py -- Multi-language support for GUI dialogs and log messages (16 languages).

unit_convert.py -- Conversion between SIMONE flow units (43 unit types).

run_pipeline.bat -- Windows batch file for automated daily operation.

## 1.2 Integration with SIMONE

The system integrates tightly with the SIMONE simulation platform:

Reading: Uses the SIMONE Python SDK (SimoneApi) to open reconstruction scenarios, enumerate consumer nodes, and read historical consumption values (Q variable).

Writing: Creates a forecast scenario (FORECAST.RDF) in the SIMONE network directory and writes predicted values using simone_write_array, with automatic unit conversion.

Detection: Auto-detects the currently open SIMONE network, available user attributes, default measurement units, and UI language.

Licensing: Detects available SIMONE licenses and scales parallel extraction accordingly.

# 2. Installation

## 2.1 Requirements

System requirements:

Python 3.10 or newer (CPython)

Windows operating system (required for SIMONE SDK and PowerShell GUI dialogs)

SIMONE V6.37e or newer with Python SDK installed

Internet access for weather data download (Open-Meteo API, free, no API key required)

Python packages (from requirements.txt):

| Package | Minimum Version | Purpose |
| --- | --- | --- |
| pandas | 2.0 | Data manipulation and Parquet I/O |
| pyarrow | 14.0 | Parquet file format support |
| lightgbm | 4.0 | Primary ML model (gradient boosting) |
| scikit-learn | 1.3 | Random Forest, Gradient Boosting, preprocessing |
| joblib | 1.3 | Model serialization |
| requests | 2.31 | HTTP client for weather API |
| numpy | 1.24 | Numerical operations |
| pyodbc | 4.0 | Database connectivity (optional) |
| psutil | 5.9 | CPU/memory monitoring for worker scaling |
| timezonefinder | 6.0 | Automatic timezone detection from GPS coordinates |
| tzdata | 2024.1 | Timezone database |

Optional packages (auto-detected, not required):

xgboost -- Alternative ML model, used if installed

timezonefinder + tzdata -- Automatic timezone detection; falls back to config if missing

## 2.2 File Placement

The forecast scripts can be placed in any directory. A typical deployment places them in the SIMONE scripts folder:

<simone_root>\scripts\forecast\

Required files in the script directory:

train.py -- Training workflow

forecast.py -- Forecast workflow

config.py -- Configuration loader

lang.py -- Translation strings

unit_convert.py -- Unit conversion tables

get_simone_network.py -- Helper for network auto-detection

run_pipeline.bat -- Batch automation (optional)

requirements.txt -- Python dependencies (for pip install; not needed with vendor/)

## 2.3 FORECAST.INI Configuration

The system can optionally read configuration from FORECAST.INI. This file is NOT required for interactive (GUI) mode — all settings are selected in the dialog. It is only needed for batch mode (--batch).

<simone_root>\sys\FORECAST.INI

The INI file is optional. All values have sensible defaults. GUI dialog settings and environment variables take priority over INI values. The INI file becomes essential for batch (unattended) operation where no GUI is shown.

Configuration priority (highest to lowest):

1. Environment variables (set by GUI dialog or external scripts)

2. FORECAST.INI in <simone_root>/sys/

3. Built-in default values in config.py

## 2.4 First-time Setup

Two installation options:

Option A — Vendor folder (recommended, no internet needed):

1. Extract forecast.zip into <simone_root>/scripts/

2. Extract forecast_vendor_cp310.zip so that vendor/ folder sits next to the scripts

3. Done — scripts auto-detect vendor/ and load packages from there

Option B — pip install (requires internet and pip):

pip install -r requirements.txt

No pip installation needed when using vendor/ folder.

Verify SIMONE SDK is accessible:

python -c "import sys; sys.path.insert(0, r'C:\Simone\Simone-V6_37e\Simone-API-SDK\Python3'); sys.path.insert(0, r'C:\Simone\Simone-V6_37e\Simone-API-SDK\Python3\Lib'); import SimoneApi; print('OK')"

# 3. Training (train.py)

The training script extracts historical gas consumption data from SIMONE reconstruction scenarios, downloads corresponding weather data, and trains a machine learning model that learns the relationship between weather conditions, time patterns, and gas consumption.

## 3.1 Starting Training

Interactive mode (with GUI dialog):

python train.py

Batch mode (reads settings from FORECAST.INI, no GUI):

python train.py --batch

In interactive mode, the script opens a Windows Forms dialog where all training parameters can be configured. In batch mode, all parameters must be pre-configured in FORECAST.INI or set via environment variables.

## 3.2 Training Dialog

The training settings dialog contains the following fields:

Network dir

Path to the SIMONE network directory. Auto-detected from the currently open SIMONE network. Use the "..." button to browse for a different directory. This determines which network's consumer nodes will be used for training.

Scenarios in

Directory to scan for RDF scenario files. Defaults to the network directory. Can be pointed to a separate archive directory containing historical reconstruction scenarios.

Subdirectories checkbox

When checked, the scanner also searches subdirectories recursively for RDF files and ZIP archives containing RDF files.

Customer type

Optional SIMONE user attribute that categorizes consumer nodes (e.g., "residential", "industrial", "commercial"). The dropdown is populated by querying SIMONE for available attribute definitions. When selected, this attribute becomes an additional feature in the ML model, improving accuracy for networks with diverse consumer types.

Weather (internet)

Country selector for automatic weather data download. Selecting a country causes the system to download temperature data for the 5 largest cities in that country from the Open-Meteo API. This provides geographic temperature diversity across the network coverage area. 47 countries are available.

or CSV

Alternative to internet download: path to a CSV file containing manual weather data. When a CSV file is selected, it overrides the internet download. See Section 8.2 for the supported CSV formats.

Extract data only checkbox

When checked, the training process only extracts scenario data to Parquet files without running customer attribute processing or model training. Weather data is still downloaded because per-node correction coefficients require yesterday's weather. This is useful for daily incremental data updates: extract today's reconstruction scenario so that tomorrow's forecast can use per-node correction coefficients calculated from today's actual consumption data. See section 3.8 for details.

## 3.3 Training Dialog Screenshot

[SCREENSHOT: Training settings dialog]

## 3.4 Extraction Process

The extraction phase processes SIMONE reconstruction scenarios to build a training dataset:

Scanning: The system scans the specified directory for .RDF files, including inside ZIP archives. Files with names containing PRSIM, ZYSIM, or LDSIM are excluded (simulation variants, not reconstructions).

Reconstruction filtering: Each scenario is checked via simone_scenario_info to verify it is a reconstruction (runtype = SIMONE_RUNTYPE_REC = 2). Non-reconstruction scenarios are skipped. This check runs in a subprocess with a 15-second timeout to prevent hangs on corrupted RDF files.

Parallel extraction: Reconstruction scenarios are extracted in parallel using ProcessPoolExecutor. Each worker process initializes its own SIMONE session (requiring a separate license). The number of workers is determined by: min(max_workers, available_licenses, 90% * CPU_count, scenario_count).

Data extraction: For each scenario, the system enumerates consumer nodes (SIMONE_OBJTYPE_NO), reads consumption values (Q variable) at all recorded timestamps using simone_read, and aggregates to hourly means.

Storage: Extracted data is saved as Parquet files (one per scenario date) in the parquet/ subdirectory. A SQLite metadata database tracks which scenarios have been extracted, enabling incremental extraction on subsequent runs.

Network resolution: When scanning across multiple directories, the system resolves each directory as a potential SIMONE network, determines consumer node overlap with the training network, and filters accordingly.

## 3.5 Weather Data

Weather data is essential for accurate forecasting, as gas consumption strongly correlates with outdoor temperature (heating demand).

Automatic download (Open-Meteo API)

The system uses the Open-Meteo API, which is free and requires no API key. For each selected country, it downloads hourly weather data for 5 cities (the largest population centers), providing geographic temperature diversity. The API provides:

Historical archive: archive-api.open-meteo.com/v1/archive

Weather forecast (48 hours): api.open-meteo.com/v1/forecast

Weather variables downloaded:

temperature_2m -- Air temperature at 2m height (primary predictor)

wind_speed_10m -- Wind speed at 10m height

relative_humidity_2m -- Relative humidity

apparent_temperature -- "Feels like" temperature (combines wind chill and humidity)

The system includes retry logic (3 attempts with 5-second delays) for HTTP 5xx errors, which are common with the free Open-Meteo tier.

Manual CSV import

Weather data can be provided via CSV file. The system also checks for a weather_manual.csv file in the data directory on every run, allowing supplemental data to be added without modifying the main configuration.

Two CSV formats are supported:

Format A (flat): datetime, temperature_1, temperature_2, ..., temperature_5, wind_speed, humidity, feels_like

Format B (row-based): datetime, location_id, temperature, wind_speed, humidity, feels_like

## 3.6 Model Training

The system uses a multi-method approach: it trains several ML models and automatically selects the best one based on validation performance.

Model candidates:

LightGBM (primary) -- Gradient boosting; fastest, usually best. Requires lightgbm package.

XGBoost -- Alternative gradient boosting. Used if xgboost is installed.

Random Forest -- Ensemble of decision trees (200 estimators, max_depth=20).

Gradient Boosting (sklearn) -- Standard gradient boosting (50 estimators, max_depth=6).

Features used:

Time features: hour, day_of_week, month, is_weekend, is_holiday, hour_sin, hour_cos, month_sin, month_cos

Weather features: temperature (5 locations), wind_speed, humidity, feels_like

Node features: node_enc (encoded node ID), q_node_mean, q_node_std, q_node_hour_mean

Optional: customer_type_enc (from SIMONE user attribute), cluster (if clustering enabled)

Validation:

The dataset is split chronologically. The validation set consists of the last N days, where N = max(1, min(7, total_days / 5)). This adaptive split ensures meaningful validation even with small datasets. Models are evaluated using RMSE (root mean squared error) and MAE (mean absolute error) on the validation set.

Feature importance:

After training, the system prints feature importance rankings for the best model, showing which factors most influence the predictions. Typically, temperature and node identity are the strongest predictors.

## 3.7 Output Files

Training produces the following files in the forecast/ data directory:

| Directory | File | Description |
| --- | --- | --- |
| models/ | lgbm_model.pkl | Trained model (joblib serialized) |
| models/ | node_encoder.pkl | LabelEncoder for node IDs |
| models/ | cluster_map.pkl | Node clustering map (if clustering enabled) |
| models/ | customer_type_encoder.pkl | Customer type encoder (if attribute used) |
| models/ | node_stats.json | Per-node mean and std statistics |
| models/ | node_hour_stats.json | Per-node per-hour mean profiles |
| parquet/ | *.parquet | Extracted scenario data (one file per date) |
|  | simone.db | SQLite database (metadata, weather, node units) |
|  | training_unit.json | Unit descriptor used during training |
|  | weather_locations.json | Weather station coordinates |
|  | train.log | Full training log |

## 3.8 Extract Only Mode

The "Extract data only" mode runs only the data extraction step of the training pipeline, skipping customer attribute processing and model training. Weather data is still downloaded because per-node correction requires yesterday's weather. Its primary purpose is incremental daily data extraction.

Purpose: In a typical automated setup, the forecast model is retrained periodically (e.g., weekly) using all available historical data. However, the per-node correction coefficient mechanism (see section 4.4, step 3) works best when yesterday's actual consumption data is available as a Parquet file. By running an extract-only job daily, today's reconstruction scenario is converted to a Parquet file without the overhead of a full retraining cycle.

How it works: When extract-only mode is active, the training script performs only these steps:

Scans the scenario directory for RDF files (same as full training).

Extracts consumption data from reconstruction scenarios in parallel and saves Parquet files.

Updates the SQLite metadata database with newly extracted scenarios.

Downloads: weather data (needed for per-node correction coefficients). Skips: node attribute processing, feature engineering, model training, and model evaluation.

This enables a workflow where extract-only runs daily (fast, typically 1-2 minutes) and full retraining runs weekly or on-demand. The daily Parquet files ensure that per-node correction coefficients in the forecast step always have access to the most recent actual data.

# 4. Forecast (forecast.py)

The forecast script generates gas consumption predictions using a previously trained model. It downloads the latest weather forecast, applies the model to produce hourly predictions, and writes the results back into SIMONE as a forecast scenario.

## 4.1 Starting Forecast

Interactive mode (with GUI dialog):

python forecast.py

Batch mode (reads settings from FORECAST.INI, no GUI):

python forecast.py --batch

## 4.2 Forecast Dialog

The forecast settings dialog contains the following fields:

Start

Start date and hour of the forecast period. Defaults to tomorrow at 00:00. Uses a date picker control and a numeric spinner for the hour (0-23).

End

End date and hour of the forecast period. Defaults to tomorrow at 23:00. The forecast generates predictions for every hour between start and end (inclusive).

Interval [min]

Output time interval in minutes. Default is 60 (hourly). The model always predicts at hourly resolution; for sub-hourly intervals, the predictions are resampled (linearly interpolated) to the requested resolution. Valid range: 1 to 1440 minutes.

Network dir

Path to the SIMONE network directory. Auto-detected from the currently open network. Must match the network used during training (same consumer node names).

## 4.3 Forecast Dialog Screenshot

[SCREENSHOT: Forecast settings dialog]

## 4.4 Forecast Process

The forecast generation follows these steps:

1. Weather download

Downloads the latest 48-hour weather forecast from Open-Meteo for all configured weather locations. This ensures the model has up-to-date weather inputs for the forecast period.

2. Model loading

Loads the trained model, node encoder, node statistics, and optional cluster map from the models/ directory.

3. Per-node correction (optional)

If yesterday's actual consumption data is available (as a Parquet file from recent training), the system computes a correction factor for each node. It compares yesterday's actual values with what the model would have predicted, and derives a damped correction factor:

Raw factor = actual_mean / predicted_mean (per node)

Clamped to [0.5, 2.0] range

Blended 50/50 with 1.0, giving effective range [0.75, 1.5]

This correction compensates for systematic model bias on a per-node basis.

4. Prediction generation

The model generates predictions at hourly resolution for all consumer nodes. Features are constructed from the forecast weather data, time features, and stored node statistics. Missing features (if the model was trained with features not available at forecast time) are filled with zeros and a warning is logged.

5. Resampling

If the requested interval is not 60 minutes, predictions are resampled. For sub-hourly intervals, linear interpolation is applied. The result is a smooth consumption profile at the desired time resolution.

## 4.5 Writing to SIMONE

After generating predictions, the system writes them into SIMONE as a forecast scenario:

Creates or overwrites FORECAST.RDF in the network directory.

Opens the scenario via simone_open in write mode.

Performs unit conversion: if the current SIMONE default unit differs from the unit used during training, values are converted using the SIMONE API (simone_write_array handles unit descriptors).

Writes consumption values for each node at each timestep using simone_write_array.

Saves and closes the scenario.

Nodes that exist in the model but not in the current SIMONE network are skipped with a warning. The unit conversion uses the full unit descriptor (unit_type << 16 | unit_code) to ensure correct interpretation by SIMONE.

## 4.6 Output Files

Forecast produces the following files:

| Directory | File | Description |
| --- | --- | --- |
| forecasts/ | forecast_YYYYMMDD_H.csv | Forecast data in CSV format |
|  | FORECAST.RDF | SIMONE scenario file (in network directory) |
|  | forecast.log | Forecast generation log |

The CSV file contains columns: datetime, node_id, q_value. It serves as a human-readable record and can be used for analysis or re-import.

# 5. Batch Mode (run_pipeline.bat)

The batch runner enables fully automated, unattended operation. It is designed to be called from Windows Task Scheduler for daily forecast generation.

## 5.1 Usage

run_pipeline.bat              -- runs both train and forecast
run_pipeline.bat train        -- runs train only
run_pipeline.bat forecast     -- runs forecast only
run_pipeline.bat all          -- runs both (same as no argument)

The batch file locates Python in PATH, determines its own script directory, and runs train.py --batch and/or forecast.py --batch. Exit codes propagate: if training fails, the forecast step is skipped.

For extract-only mode, pass the --extract-only flag: python train.py --batch --extract-only. Alternatively, set extract_only=true in the [training] section of FORECAST.INI. This runs only the data extraction step with weather download but without customer attribute processing or model training.

## 5.2 FORECAST.INI Sections for Batch Mode

The following INI sections must be configured for unattended operation:

[training]

scan_dir -- Directory containing reconstruction scenarios

include_subdirs -- Whether to scan subdirectories (true/false)

customer_attribute -- Name of SIMONE user attribute for customer type (optional)

extract_only -- Extract scenario data only, skip customer attributes and training (weather is still downloaded) (true/false)

[forecast]

start -- Start date/hour (e.g., "tomorrow 0" or "2025-01-15 0")

end -- End date/hour (e.g., "tomorrow 23" or "2025-01-15 23")

interval_min -- Output interval in minutes (default: 60)

[weather]

country -- Country code for weather download (e.g., CZ, DE, AT)

weather_csv -- Alternative: path to CSV file with weather data

## 5.3 Example FORECAST.INI for Automated Daily Operation

[simone]
root = C:\Simone\Simone-V6_37e
network = ICGB_LIVE_20231220
scenario_dir = C:\Simone\Simone-V6_37e\nets\ICGB_LIVE_20231220

[weather]
country = CZ

[training]
scan_dir = C:\Simone\Simone-V6_37e\nets\ICGB_LIVE_20231220
include_subdirs = false

[forecast]
start = tomorrow 0
end = tomorrow 23
interval_min = 60

[parallel]
max_workers = 4

# 6. Configuration Reference

Complete reference of all FORECAST.INI keys:

| Section | Key | Default | Description |
| --- | --- | --- | --- |
| simone | root | C:\Simone\Simone-V6_37e | SIMONE installation root directory |
| simone | network | (auto-detect) | SIMONE network name |
| simone | scenario_dir | <root>/nets/<network> | Directory containing scenario RDF files |
| simone | scenario_ext | .RDF | Scenario file extension |
| simone | consumer_objtype_flag | SIMONE_OBJTYPE_NO | SIMONE object type flag for consumer nodes |
| simone | consumption_var_suffix | .Q | Variable suffix for consumption (flow) reading |
| weather | lat | 49.75 | Latitude for single-location weather (legacy) |
| weather | lon | 15.5 | Longitude for single-location weather (legacy) |
| weather | vars | temperature_2m,... | Open-Meteo hourly variables to download |
| weather | tz_offset | 2 | UTC offset for SIMONE timestamps (hours) |
| weather | max_locations | 5 | Maximum number of weather locations |
| weather | country | (none) | Country code for auto 5-city weather (batch mode) |
| weather | weather_csv | (none) | Path to CSV file with manual weather data |
| training | scan_dir | (network dir) | Directory to scan for RDF scenarios |
| training | include_subdirs | false | Scan subdirectories for RDF files |
| training | customer_attribute | (none) | SIMONE user attribute name for customer type |
| training | extract_only | false | Extract scenario data only, skip weather download and model training |
| forecast | start | tomorrow 0 | Forecast start (date + hour) |
| forecast | end | tomorrow 23 | Forecast end (date + hour) |
| forecast | interval_min | 60 | Output time interval in minutes |
| paths | data_dir | <scenario_dir>/forecast | Root directory for all forecast data |
| model | clustering | false | Enable K-Means node clustering |
| model | n_clusters | 20 | Number of clusters (if clustering=true) |
| model | objective | regression | LightGBM objective function |
| model | metric | rmse | LightGBM evaluation metric |
| model | learning_rate | 0.05 | LightGBM learning rate |
| model | num_leaves | 63 | LightGBM max leaves per tree |
| model | min_data_in_leaf | 20 | LightGBM min samples per leaf |
| model | n_estimators | 500 | Number of boosting iterations |
| model | lag_hours | 1,2,3,24,48,168 | Lag feature hours (comma-separated) |
| parallel | max_workers | 10 | Maximum parallel extraction workers (batch mode only; in interactive mode, workers are auto-calculated based on available SIMONE licenses and CPU cores) |

Environment variable overrides:

| Environment Variable | Overrides |
| --- | --- |
| SIMONE_ROOT_OVERRIDE | [simone] root |
| SIMONE_NETWORK_NAME | [simone] network |
| SIMONE_NETWORK_DIR | [simone] scenario_dir |
| FORECAST_DATA_DIR | [paths] data_dir |
| FORECAST_MAX_WORKERS | [parallel] max_workers |
| WEATHER_LAT | [weather] lat |
| WEATHER_LON | [weather] lon |
| SIMONE_TZ_OFFSET | [weather] tz_offset |

# 7. Multi-language Support

The SIMONE Forecast System supports 16 languages for GUI dialogs, log messages, country names, and status output. The language is automatically detected from the SIMONE installation.

## 7.1 Supported Languages

| Code | Language |
| --- | --- |
| us | English (US) -- default |
| en | English (alias for us) |
| cz | Czech (Cestina) |
| de | German (Deutsch) |
| pl | Polish (Polski) |
| hu | Hungarian (Magyar) |
| f | French (Francais) |
| it | Italian (Italiano) |
| e | Spanish (Espanol) |
| hr | Croatian (Hrvatski) |
| ru | Russian |
| sl | Slovenian (Slovenscina) |
| uk | Ukrainian |
| p | Portuguese (Portugues) |
| jp | Japanese |
| cn | Chinese (Simplified) |

## 7.2 Language Detection

Language is detected automatically through the following mechanism:

Primary: The SIMONE API function simone_get_config_item is called with section "logical_names" and key "LANG_ID" to read the SIMONE installation language.

Fallback: If the API call fails, the system falls back to the FORECAST.INI configuration or defaults to English (us).

All GUI dialog labels, button texts, log messages, error messages, and country names are translated. The translation function t(key, **kwargs) supports dynamic placeholders (e.g., {n}, {path}) that are filled at runtime.

## 7.3 Country Names

Country names are translated into 11 languages: English, Czech, German, French, Spanish, Russian, Polish, Italian, Japanese, and Chinese. Other languages fall back to English names.

# 8. Weather Data

Weather data is the primary external input for the forecast model. Gas consumption is strongly correlated with outdoor temperature (lower temperatures increase heating demand), and the model also uses wind speed, humidity, and apparent temperature as features.

## 8.1 Automatic Download (Open-Meteo API)

The system uses the Open-Meteo API, a free weather data service that requires no API key or registration. Two endpoints are used:

Historical archive: https://archive-api.open-meteo.com/v1/archive -- provides hourly historical data for any date range and location.

Forecast: https://api.open-meteo.com/v1/forecast -- provides 48-hour weather forecasts.

For each country selected in the training dialog (or configured in [weather] country), the system downloads data for the 5 largest cities, providing temperature measurements spread across the gas network's coverage area. These become model features temperature_1 through temperature_5.

## 8.2 Manual CSV Format

Two CSV formats are supported for manual weather data import:

Format A: Flat (one row per timestamp)

datetime,temperature_1,temperature_2,temperature_3,temperature_4,temperature_5,wind_speed,humidity,feels_like
2025-01-15T00:00,2.1,1.8,3.2,0.5,1.9,12.5,78,0.3
2025-01-15T01:00,1.9,1.5,2.8,0.2,1.7,11.8,80,-0.1

Format B: Row-based (one row per location per timestamp)

datetime,location_id,temperature,wind_speed,humidity,feels_like
2025-01-15T00:00,0,2.1,12.5,78,0.3
2025-01-15T00:00,1,1.8,,,

Additionally, the system checks for a file named weather_manual.csv in the data directory on every run. This allows continuous supplementation of weather data without changing any configuration.

## 8.3 Supported Countries

The following 47 countries are available for automatic weather data download:

| Code | Country | Code | Country | Code | Country |
| --- | --- | --- | --- | --- | --- |
| CZ | Czechia | MT | Malta | LT | Lithuania |
| SK | Slovakia | HR | Croatia | LV | Latvia |
| DE | Germany | RS | Serbia | EE | Estonia |
| AT | Austria | BA | Bosnia and Herzegovina | RU | Russia |
| CH | Switzerland | ME | Montenegro | KZ | Kazakhstan |
| PL | Poland | MK | North Macedonia | UZ | Uzbekistan |
| HU | Hungary | AL | Albania | CN | China |
| SI | Slovenia | XK | Kosovo | JP | Japan |
| LU | Luxembourg | RO | Romania | TH | Thailand |
| FR | France | BG | Bulgaria | ID | Indonesia |
| BE | Belgium | MD | Moldova | IR | Iran |
| NL | Netherlands | TR | Turkey | SA | Saudi Arabia |
| GB | United Kingdom | DK | Denmark | AE | UAE |
| IE | Ireland | NO | Norway | EG | Egypt |
| ES | Spain | SE | Sweden | DZ | Algeria |
| PT | Portugal | FI | Finland | MA | Morocco |
| IT | Italy | IS | Iceland | AU | Australia |
| GR | Greece | UA | Ukraine |  |  |
| CY | Cyprus | BY | Belarus |  |  |

## 8.4 Weather Variables Used

| Variable | API Name | Unit | Description |
| --- | --- | --- | --- |
| Temperature | temperature_2m | deg C | Air temperature at 2m height above ground |
| Wind speed | wind_speed_10m | km/h | Wind speed at 10m height |
| Humidity | relative_humidity_2m | % | Relative humidity at 2m height |
| Feels like | apparent_temperature | deg C | Apparent temperature (wind chill + humidity) |

# 9. Troubleshooting

Common errors and their solutions:

"No consumer nodes found"

The SIMONE network does not contain any consumer nodes (object type SIMONE_OBJTYPE_NO). Verify that the correct network directory is selected and that the network contains offtake nodes. Check the consumer_objtype_flag setting in FORECAST.INI if using a non-standard network configuration.

Scenario extraction timeout (15 seconds)

The reconstruction check subprocess timed out. This typically indicates a corrupted RDF file that causes the SIMONE API to hang. The scenario is skipped automatically. If many scenarios time out, check for corrupted files in the scenario directory.

Weather API HTTP 502 errors

The Open-Meteo API returned a server error. The system includes automatic retry logic: 3 attempts with 5-second delays between retries. If the error persists, the API may be temporarily unavailable. Use a manual CSV file as a workaround, or retry later.

"Warning: missing features ... filling with zero"

The model was trained with features that are not available at forecast time. This can happen when the model was trained with customer_type_enc but the attribute is not available during forecasting, or when weather data columns differ. The missing features are filled with zeros, which may reduce forecast accuracy. Retrain the model to resolve.

WinError 183 (file rename)

Windows error "Cannot create a file when that file already exists." This was a known issue with os.rename on Windows. The system now uses os.replace, which handles existing files correctly. If this error occurs, update to the latest version of the scripts.

"No Parquet files in ..."

The forecast script cannot find training data. Run train.py first to extract SIMONE data and generate Parquet files. Verify the FORECAST_DATA_DIR or [paths] data_dir configuration points to the correct directory.

"Model not found"

The forecast script cannot find the trained model file (lgbm_model.pkl). Run train.py first. Check that the models/ subdirectory exists in the data directory and contains the model file.

License detection issues

The system probes for available SIMONE licenses by spawning test processes. If license detection fails or reports fewer licenses than expected, check that SIMONE licenses are properly configured and that no other processes are holding licenses.

Timezone mismatch

If forecast timestamps appear shifted, check the tz_offset setting in [weather]. The system attempts automatic timezone detection using the timezonefinder package based on weather location GPS coordinates. If the package is not installed, it falls back to the configured offset (default: UTC+2).

# Appendix A: File Structure

Complete list of files in the SIMONE Forecast System:

Script files:

| File | Description |
| --- | --- |
| train.py | Main training workflow: extraction, weather, model training |
| forecast.py | Main forecast workflow: prediction, SIMONE write-back |
| config.py | Configuration: INI loading, env overrides, defaults |
| lang.py | Multi-language translations (16 languages, ~200 keys each) |
| unit_convert.py | SIMONE unit conversion (43 flow units) |
| get_simone_network.py | Helper: detects currently open SIMONE network |
| run_pipeline.bat | Windows batch runner for automated operation |
| requirements.txt | Python package dependencies |
| FORECAST.INI | Configuration file (placed in <simone_root>/sys/) |

Generated data files (in forecast/ data directory):

| Path | Description |
| --- | --- |
| models/lgbm_model.pkl | Trained ML model |
| models/node_encoder.pkl | Node ID label encoder |
| models/cluster_map.pkl | K-Means cluster assignments (optional) |
| models/customer_type_encoder.pkl | Customer type encoder (optional) |
| models/node_stats.json | Per-node consumption statistics (mean, std) |
| models/node_hour_stats.json | Per-node per-hour average profiles |
| parquet/*.parquet | Extracted scenario data (one file per date) |
| forecasts/forecast_*.csv | Generated forecast CSV files |
| simone.db | SQLite metadata database |
| training_unit.json | Unit descriptor from training |
| weather_locations.json | Weather station coordinates |
| customer_attr_values.json | Customer attribute values (if used) |
| train.log | Training log |
| forecast.log | Forecast log |

# Appendix B: Supported Countries

Complete list of 47 countries supported for automatic weather data download. For each country, temperature data is downloaded for the 5 largest cities.

| Code | Country | Cities (5 largest) |
| --- | --- | --- |
| AL | Albania | Tirana, Durres, Vlore, Elbasan, Shkoder |
| AT | Austria | Wien, Graz, Linz, Salzburg, Innsbruck |
| AU | Australia | Sydney, Melbourne, Brisbane, Perth, Adelaide |
| AE | UAE | Dubai, Abu Dhabi, Sharjah, Ajman, Ras al-Khaimah |
| BA | Bosnia and Herzegovina | Sarajevo, Banja Luka, Tuzla, Zenica, Mostar |
| BE | Belgium | Bruxelles, Antwerpen, Gent, Charleroi, Liege |
| BG | Bulgaria | Sofia, Plovdiv, Varna, Burgas, Ruse |
| BY | Belarus | Minsk, Gomel, Mogilev, Vitebsk, Grodno |
| CH | Switzerland | Zurich, Geneve, Basel, Bern, Lausanne |
| CN | China | Beijing, Shanghai, Guangzhou, Shenzhen, Chengdu |
| CY | Cyprus | Nicosia, Limassol, Larnaca, Paphos, Famagusta |
| CZ | Czechia | Praha, Brno, Ostrava, Plzen, Liberec |
| DE | Germany | Berlin, Munchen, Hamburg, Koln, Frankfurt |
| DK | Denmark | Kobenhavn, Aarhus, Odense, Aalborg, Frederiksberg |
| DZ | Algeria | Alger, Oran, Constantine, Annaba, Blida |
| EE | Estonia | Tallinn, Tartu, Narva, Parnu, Kohtla-Jarve |
| EG | Egypt | Cairo, Alexandria, Giza, Port Said, Suez |
| ES | Spain | Madrid, Barcelona, Valencia, Sevilla, Bilbao |
| FI | Finland | Helsinki, Espoo, Tampere, Turku, Oulu |
| FR | France | Paris, Marseille, Lyon, Toulouse, Nice |
| GB | United Kingdom | London, Birmingham, Manchester, Glasgow, Leeds |
| GR | Greece | Athina, Thessaloniki, Patra, Iraklion, Larisa |
| HR | Croatia | Zagreb, Split, Rijeka, Osijek, Zadar |
| HU | Hungary | Budapest, Debrecen, Szeged, Miskolc, Pecs |
| ID | Indonesia | Jakarta, Surabaya, Bandung, Medan, Semarang |
| IE | Ireland | Dublin, Cork, Galway, Limerick, Waterford |
| IR | Iran | Tehran, Mashhad, Isfahan, Tabriz, Shiraz |
| IS | Iceland | Reykjavik, Kopavogur, Hafnarfjordur, Akureyri, Reykjanesbaer |
| IT | Italy | Roma, Milano, Napoli, Torino, Palermo |
| JP | Japan | Tokyo, Yokohama, Osaka, Nagoya, Sapporo |
| KZ | Kazakhstan | Almaty, Astana, Shymkent, Karaganda, Aktobe |
| LT | Lithuania | Vilnius, Kaunas, Klaipeda, Siauliai, Panevezys |
| LU | Luxembourg | Luxembourg, Esch-sur-Alzette, Differdange, Dudelange, Ettelbruck |
| LV | Latvia | Riga, Daugavpils, Liepaja, Jelgava, Jurmala |
| MA | Morocco | Casablanca, Rabat, Fes, Marrakech, Tanger |
| MD | Moldova | Chisinau, Balti, Cahul, Ungheni, Soroca |
| ME | Montenegro | Podgorica, Niksic, Bijelo Polje, Bar, Herceg Novi |
| MK | North Macedonia | Skopje, Bitola, Kumanovo, Prilep, Ohrid |
| MT | Malta | Valletta, Birkirkara, Mosta, Qormi, Sliema |
| NL | Netherlands | Amsterdam, Rotterdam, Den Haag, Utrecht, Eindhoven |
| NO | Norway | Oslo, Bergen, Trondheim, Stavanger, Tromso |
| PL | Poland | Warszawa, Krakow, Lodz, Wroclaw, Poznan |
| PT | Portugal | Lisboa, Porto, Braga, Coimbra, Faro |
| RO | Romania | Bucuresti, Cluj-Napoca, Timisoara, Iasi, Constanta |
| RS | Serbia | Beograd, Novi Sad, Nis, Kragujevac, Subotica |
| RU | Russia | Moskva, Sankt-Peterburg, Novosibirsk, Yekaterinburg, Kazan |
| SA | Saudi Arabia | Riyadh, Jeddah, Mecca, Medina, Dammam |
| SE | Sweden | Stockholm, Goteborg, Malmo, Uppsala, Linkoping |
| SI | Slovenia | Ljubljana, Maribor, Celje, Kranj, Koper |
| SK | Slovakia | Bratislava, Kosice, Presov, Zilina, Banska Bystrica |
| TH | Thailand | Bangkok, Chiang Mai, Pattaya, Nakhon Ratchasima, Chon Buri |
| TR | Turkey | Istanbul, Ankara, Izmir, Bursa, Antalya |
| UA | Ukraine | Kyiv, Kharkiv, Odesa, Dnipro, Lviv |
| UZ | Uzbekistan | Tashkent, Samarkand, Bukhara, Namangan, Andijan |
| XK | Kosovo | Prishtina, Prizren, Peja, Mitrovica, Gjilan |